Powder spreading and spreadability in the additive manufacturing of metallic materials: A critical review

(https://doi.org/10.1016/j.jmatprotec.2022.117706)

Description of additive manufacturing (AM) processes

- Powder-based AM

  - Powder bed fusion (PBF)

    - Localized heat source that melts/sinters powders that are previously spread over a build platform

    - Progresses layer by layer with a new layer of powder being spread between each time

    - Includes selective laser sintering (SLS), selective laser melting (SLM), and electron beam melting (EBM)

    - SLM offers more accurate parts and better surface quality, EBM is faster and produces components with lower residual stresses

    - In general, laser bed tech introduced the development of new alloys made for the AM field

  - Directed Energy Deposition (DED)

    - Introduces a metal powder to a laser/electron beam/plasma arc/general heat source to melt powders together

    - A little less relevant to the characteristics we test

    - Often used to repair parts independent of surface geometry and in particular is used in the aerospace industry

  - Binder Jetting (BJ)

    - Layered printing where in between each layer of metal powder is binder droplets