Preliminary Design Report - Verification of the Flat Plate Convection Model

Heat transfer is foundational in the modern world, with some applications including manufacturing, cooking, electronics, HVAC, and transportation. The methods of heat transfer - conduction, convection, and radiation - are so critical that a combination of these enables human life itself, by allowing the body to regulate its temperature in changing environmental conditions. The goal of this project is to isolate and investigate one of these methods of heat transfer, natural convection. This will be done by constructing a thermally isolated enclosure around a hot surface/plate which minimizes external airflow and other sources of heat transfer so that heat flow can be measured and compared to predicted convective heat transfer values using the flat plate convection model. A preliminary CAD assembly of the enclosure and project setup can be seen in Figure 1 below.

The experiment will also investigate a quantity of interest, the convection coefficient (h), which, according to the flat plate convection model, is defined as:

h = qconv''Ts-T∞ #1

Where q'' [W/m2] is the heat flux, Ts [K] is the surface temperature of the hot plate, and T∞ [K] is the ambient temperature of the box. As Ts - T∞ represents a temperature difference, with a flat conversion from K to °C, °C can also be used in Equation 1.

Generally, “convection is the mode of energy transfer between a solid surface and [an] adjacent liquid or gas that is in motion, and it involved the combined effects of conduction and fluid motion” [1]. Unlike forced convection, which uses external means to force fluid flow over a surface, natural convection occurs when the fluid motion is due to buoyancy forces induced by temperature-dependent density discrepancies. In other words, according to the model, in our experiment, conduction is expected at the surface of the hot plate between the hot solid surface and the cooler air. This heat transfer should increase the temperature of the air along the surface, decreasing its density and causing it to rise towards the top of the enclosure. As this air rises away from the hot-plate surface, it is immediately replaced by cooler air and the process repeats, generating a net airflow away from the hotplate and therefore the fluid motion required for convection.

Three sensors will be used to measure the heat flux from the hot plate, each corresponding to one value on the right-hand side of Equation 1. A heat flux sensor (FluxTeq PHFS-01) will be used to measure q'', a peel-and-stick type K thermocouple (Watlow OKK30B10A) will be placed directly on the hot plate to measure Ts, and a platinum resistance temperature detector (RTD) (Adafruit PT100) will be used to measure the ambient box temperature, T∞. Data will be collected using a

It was ensured that each of these sensors would be within their operating ranges during experimentation. The preliminary experimental conditions are as follows: T∞ corresponds to the initial ambient room temperature of the lab, approximately 25°C, Ts is effectively the only independent variable which is able to be varied as needed and will be initially set to 60°C, q'' is a function of h, Ts, and T∞ and is expected to be about 350W/m², corresponding to a convection coefficient of 10

An important consideration when trying to isolate natural convection is assessing and minimizing the effects of other heat transfer modes. Forced convection will be assumed to be negligible as any forced airflow is minimal due to the enclosure’s design. Non-natural convection -related conduction will also be assumed to be negligible as the hot plate, and the heat flux sensor attached directly on its surface, will be assumed to both be at Ts and will only be in contact with the enclosed air.

Radiation is the final mode of heat transfer to consider and will not be neglected. According to [2], “radiation that is emitted by the surface originates from the thermal energy of matter bounded by the surface and the rate at which energy is released per unit area (W/m2) is termed the surface emissive power, E.” This energy is released in the form of heat, and an ideal radiator or blackbody emits the heat flux, Eb, as shown in Equation 2.

Eb = σTs4						(2a)

E=qrad'' = εσTs4					       (2b)

Where σ is the Stefan-Boltzmann constant (σ = 5.67 x 10-8 Wm2K) and Ts is the absolute surface temperature in Kelvin. As the hot plate is not an ideal radiator, Equation 2a is scaled by an emissivity value, ε, which is between 0 and 1 and varies on properties such as material and finish (see Equation 2b). For aluminum, the hot-plate surface material, this value is often between about 0.03 to 0.20, depending on surface polish or oxidation level, but can be as high as 0.84 for anodized aluminum [2]. Because the exact quality of the surface is unknown, the hot plate will be covered in a layer of known low-emissivity aluminum foil (ε = 0.03). So, at Ts = 60°C = 333.15 K, qrad'' is expected to be ~21 W/m2, much smaller than the expected 350 W/m2 due to natural convection, but not negligible. Therefore, to assess natural convection, the heat flux due to radiation will be subtracted out of the total heat flux measured by the heat flux sensor, as shown in Equation 3.

qconv'' = qmeasured''-qrad''			          (3)

The advantages of this experimental setup are its simplicity, low cost, ease of modification, and reliability. The enclosure’s structure is built using two widely available and low-cost materials with great thermal insulation properties, Styrofoam and cardboard. Cost is also low as all sensors and measurement devices needed are already owned and available. The enclosures shape is an easily constructed box, and any dimension changes or adjustment can be made quickly with little to no cost, assuming extra materials are available. Any gaps in the enclosure will be easily and effectively sealed using duct seal putty, ensuring that external environmental influences are minimal within the enclosure. Lastly, the design itself is effective at isolating natural convection by creating a thermally insulated environment, isolated from outside air flow and disturbances.

Some disadvantages and perceived challenges of this design include a lack of translatable and applicable results and an inability to completely isolate natural convection as the sole heat transfer mechanism. Firstly, as the experimental conditions are quite unique and isolated, they are not very applicable to general situations. Though we expect to determine the convection coefficient of air, this value is dependent on a variety of factors, including hot plate orientation, flow characteristics, surface geometry, and “an assortment of fluid thermodynamic and transport properties” [2]. The conditions of our experiment are unique, and the isolated nature of the hot plate within the enclosure is unlikely to be useful for general applications. Additionally, if we wanted to vary conditions and examine how the convection coefficient changes, such as if the hot plate were vertical as opposed to horizontal, this could not be readily done. Lastly, a potential challenge is a changing and irregular temperature gradient within the enclosure. In other words, T∞ is likely to change during testing, and is expected to vary with distance from the hot plate, leading to changes in heat flux from the hot plate, and variation in the convection coefficient throughout the test. Although, because these variations are expected by the model of natural convection, while they might inhibit our ability to measure a single convection coefficient value, observing these variations would be beneficial in confirming the model of natural convection.

Selected Sensors

Ambient Temperature Sensor – PT100 RTD with MAX31865 Amplifier:

The ambient air temperature T∞ is measured using a Platinum Resistance Temperature Detector (PT100 RTD) paired with a MAX31865 Amplifier. The RTD is a highly accurate temperature sensor with a resistance of 100 Ω  at 0° and a temperature coefficient of about 0.385 ohms per degree Celsius. The measurement range spans from about -200° Celsius to +850° . This range significantly exceeds our expected lab range ambient range from 15-35 ° . The tolerance of the PT100 RTD is ±0.30 degrees Celsius at 0° , increasing to ±0.50°  at about 40° , and ±0.80°  at 100° . This level of precision makes the PT100 the preferred option for accurate ambient temperature measurement.

The MAX31865 breakout board has a precise 430-ohm reference resistor with 0.1% tolerance. It communicates with the Arduino over SPI, which converts the RTD resistance to a temperature value. The MAX31865 supports 2-, 3-, and 4-wire RTD configurations. This compensates for lead resistance, which is important for gaining accurate low-temperature measurements. When we combine the PT100 with the MAX31865, the system achieves a temperature resolution of about 0.03°C. This provides sufficient precision for the ambient temperature in our experiment.

Mounting: The PT 100 probe will be mounted on a thin support wire in the enclosed box, positioned far enough from the heat plate to avoid interference from the plate's thermal field. The MAX31865 board will be on the outside of the enclosure, on a breadboard, and connected to the Arduino that we will use.

Surface Temperature Sensor – Watlow OKK30B10A peel-and-stick Type K thermocouple:

This sensor will be used to measure the surface temperature of the hot plate. The specific operating range and uncertainties of this sensor were not found, although the thermocouple is described as suitable for continuous use at 200 °C or limited use up to 260 °C. This ensures that the high-end limit of this sensor is more than sufficient for our purposes. Because a specific bottom-end of the operating range could not be found, the lab teaching assistants will be consulted to ensure that this sensor is suitable. However, this sensor is very likely to be functional within the limited expected temperature range of 25°C to 60°C.

The uncertainty used for analysis was the standard of Type K thermocouples, 0.75% or ±2°C. This is a larger uncertainty than the Platinum RTD, but some of this uncertainty can hopefully be accounted for via calibration.

Mounting: The mounting method and flat profile of this sensor is what makes this sensor quite useful and applicable for our application. This flat sensor will simply use the peel-and-stick attachment of the sensor directly onto the foil overlapping the hot plate, near the heat flux sensor.

Heat Flux Sensor – FluxTeq PHFS-01 heat flux sensor with FluxTeq FluxDAQ and computer with FluxDAQ program to acquire data:

The main sensor we will use is the FluxTeq PHFS-01. This sensor has a normal sensitivity of about 7.7 mV/(W/cm^2) and works within a temperature range of -50 to +120°C, and it has a measured range of +-150 kW/m^2. The sensor has a step input response time of about 0.6 seconds.

This sensor has a small output individually, typically a few millivolts, necessitating the usage of a DAQ to amplify and record the signal. As described in [3], the heat transfer from the hot plate to the surrounding air can be found using Equation 4:

qmeasured'' = Vout1k106μVV				(2a)

where qmeasured'' is the measured rate of heat transfer per unit area from the hot plate to the surrounding air [W/m^2], Vout is the recorded output voltage of the DAQ [V], and k is the sensitivity of the heat flux sensor μVW/m2.

Mounting: This sensor will be attached to the top of the hot plate, on top of the foil, next to the Ts sensor. It will be attached using adhesive, likely masking tape.

Uncertainty Analysis of Design

To evaluate the uncertainty in the calculated convection coefficient, it is necessary to quantify the measurement uncertainties associated with each sensor used in the experiment. The primary measured quantities are heat flux q’’, surface temperature Ts, and ambient temperature T∞, each obtained using different sensors.

A representative operating condition is selected: Ts = 60°C, T∞ = 25°C, q” = 350W/m². These values fall within the expected experimental range and yield a convection coefficient within the typical natural convection range.

The heat flux sensor does not specify an explicit accuracy; therefore, a conservative uncertainty of ±5% of the reading is assumed. The RTD is used to measure the ambient temperature, and its uncertainty is calculated using the Class A tolerance equation. The thermocouple is used to measure the surface temperature, with an uncertainty of ±2°C. Detailed operating procedures for the peel-and-stick sensor are not fully specified in the available documentation. These will be clarified with the lab TA prior to testing to ensure proper installation, calibration, and data acquisition.

All sensor uncertainties are assumed to be independent and random, allowing the use of standard uncertainty propagation.

In addition, radiative heat transfer is estimated using the Stefan-Boltzmann relation and treated as a correction to the measured heat flux. Based on the use of aluminum foil with low emissivity (ε ≈ 0.03), the radiative heat flux is estimated to be approximately 20 W/m2. A design-stage uncertainty of ±5 W/m2 is assigned to this term.

Table 1: Uncertainty Parameter

| Variables | Sensor | Measured Value (Design) | Uncertainty Expression | Estimated Uncertainty Ux |
| --- | --- | --- | --- | --- |
| Heat flux q′′ | Heat flux sensor | 350 W/m² | ±5% | ±17.5 W/m² |
| Surface temp Ts | Thermocouple | 60°C | ±2°C | ±2°C |
| Ambient temp T∞ | RTD | 25°C | ±(0.15 + 0.002T) | ±0.20°C |
| Radiation q′′ rad | / | 20 W/m² | / | ±5 W/m² |

The convection coefficient for the experiment is defined as:

h = qmeasured''-qrad''Ts-T∞

where q′′ is the measured heat flux, q′′rad is the radiative heat flux, Ts is the hot surface temperature, and T∞ is the ambient air temperature.

The uncertainty in h is estimated using standard propagation of uncertainty:

Uh=∂h∂q''Uq''2+∂h∂qrad''Uqrad''2+∂h∂TsUTs2+∂h∂T∞UT∞2

The required partial derivatives are:

∂h∂q''=1Ts-T∞

∂h∂qrad''=-1Ts-T∞

∂h∂Ts=-q''Ts-T∞2

∂h∂T∞=q''Ts-T∞2

Substituting these into the propagation equation gives:

Uh=Uq''Ts-T∞2+-q''UTsTs-T∞22+q''UT∞Ts-T∞22

This expression shows that the uncertainty in h depends on both heat flux and temperature measurements and becomes increasingly sensitive as the temperature difference decreases.

For the selected condition, the temperature difference is:

ΔT=Ts-T∞=35°C

The uncertainty in the temperature difference is estimated as:

UΔT=UTs2+UT∞2=22+0.22≈2.01°C

This indicates that the uncertainty in ΔT is dominated by the surface temperature measurement.

And the expected convection coefficient is:

h=350-2035=9.43 Wm2⋅K

Substituting into the uncertainty equation:

Uh=17.5352+5352+-33023522+3500.23522≈0.73Wm2⋅K

Therefore, the design-stage estimate for the measured convection coefficient is:

h=9.43±0.73 Wm2⋅K

Which corresponds to a relative uncertainty of 7.7%.

The predicted uncertainty is acceptable for a preliminary natural convection experiment. The dominant contribution arises from the surface temperature measurement, indicating that the thermocouple is the primary limitation in the current design. In addition, the analysis shows that uncertainty increases rapidly as the temperature difference decreases, reinforcing the need to operate at sufficiently large Ts −T∞. Overall, the proposed setup is suitable for initial validation, with surface temperature measurement identified as the main source of uncertainty.

Other Supplies and Fixtures

Duct seal putty will be utilized to seal any holes or gaps around the box layers’ circular insertion points for the sensor wires and the top side mount for the ambient temperature RTD. Since there is a possibility that the wires and the RTD will need to be repositioned during testing, a surplus amount of putty will be kept in reserve. A breadboard will also be used for connecting the temperature sensors, amplifiers, and the Arduino microcontroller. Special care will be taken to ensure that connections remain secure, and that unintentional debris accumulation in the breadboard is avoided. To maximize the isolation of the enclosure from the surroundings aluminum or tin foil will be utilized for the purpose of reflecting any radiation towards the inside of the box to minimize any heat transfer exchange from the inside of the enclosure to the surroundings. The foil will also be used to minimize radiation emission from the hot plate. Sheets of foil will be carefully measured and torn so that the number of separate sheets is minimized, and non-uniform heat transfer related to discontinuities between edges/overlapping sheets are reduced. Styrofoam will also be integrated in the design for the purpose of lowering the thermal conductivity of the box layers so that conduction from the plate to the box and from the box to the outside is minimized and convective heat transfer is isolated. Like the foil, Styrofoam pieces will be carefully cut and attached to each other so that the number of separate pieces are minimized, and non-uniform heat transfer related to discontinuities between piece edges or overlapping pieces is reduced.

Bill Of Materials:

| Item | Model | Quantity | Est. Unit Cost | Est. Total | Source |
| --- | --- | --- | --- | --- | --- |
| Ambient Temp. Sensor (RTD) | PT100 Probe (3 wires) | 1 | Lab available | _ | https://www.adafruit.com/product/3290 (Lab) |
| RTD Amplifier Board | Adafruit MAX31865 | 1 | Lab available | _ | https://www.adafruit.com/product/3328 (Lab) |
| Hot Plate (Heat Source) | MEMS 1042 lab hot plate (Aluminum) | 1 | Lab available | _ | Lab |
| Heat Flux Sensor | FluxTeq PHFS-01 | 1 | Lab available | _ | https://www.fluxteq.com/phfs-01e-heat-flux-sensor (Lab) |
| Heat Flux DAQ System | FluxTeq FluxDAQ | 1 | Lab available | _ | https://www.fluxteq.com/phfs-01e-heat-flux-sensor (Lab) |
| Surface Temp. Thermocouple | WATLOW OKK30B10A Type-K peel-and-stick thermocouple | 1 | Lab available | _ | Lab |
| Type-K Thermocouple Amplifier | Adafruit MAX31855 | 1 | Lab available |  | Lab |
| Micro-controller | Arduino Uno Rev3 | 1 | Lab available | _ | Lab |
| Breadboard | generic 400-pt breadboard | 1 | Lab available | _ | Lab |
| DC Power Supply | Lab DC Power Supply | 1 | Lab available | _ | Lab |
| Lab Computer and USB Cable | Windows PC | 1 | Lab available | _ | Lab |
| Temp. Sensor Calibrator | Transformation 1064P | 1 | Lab available | _ | Lab |
| Thermocouple Amplifier | MAX31855 breakout board | 1 | Lab available | _ | https://www.adafruit.com/product/269 (Lab) |
| Pin-to-pin jumper wires | - | 15 | Lab available | - | Lab |
| Duct Seal Putty | DAP Duct Seal | 1 | ~$10 | ~$10 | Lab or hardware store, Amazon |
| Aluminum / Tin Foil | Heavy-duty Aluminum foil | 1 roll | ~$10 | ~$10 | Walmart, Amazon |
| Cardboard | Corrugated cardboard | 1 box | ~$5 | ~$5 | Recycled / Hardware, Walmart |
| Styrofoam Sheets | Generic rigid foam | 3 sheets | ~$10 each | ~$30 | Hardware store, Lowes, Home Depot |

References:

[1] Cengel and Boles, Thermodynamics: An Engineering Approach, 5th edition, McGraw-Hill, 2006.

[2] Bergman, Lavine, Incropera, and Dewitt, Fundamentals of Heat and Mass Transfer, 7th Edition, John Wiley and Sons, 2011

[3] Vipperman, J., “Lab 2 Handout - Insulation R-Value of Single-Pane and Dual-Pane

Windows”, University of Pittsburgh, 2026.