The motor we choose is 75:1 Micro Metal Gearmotor HP 6V with Extended Motor Shaff

https://www.pololu.com/product/2215

For a brushed DC motor operating at a fixed supply voltage, the torque–speed relationship can be approximated as linear. As discussed in lecture, the torque–speed curve can be modeled as a straight line （y = mx + b） defined by two datasheet endpoints:

1. Stall torque 𝑇𝑠𝑡𝑎𝑙𝑙 at zero speed （n = 0）

2. No-load speed 𝑛𝑛𝑜 𝑙𝑜𝑎𝑑at zero torque ( 𝑇 = 0)

Therefore, the motor torque available at speed n can be expressed as:

𝑇𝑛=𝑇𝑠𝑡𝑎𝑙𝑙1−𝑛𝑛𝑛𝑜 𝑙𝑜𝑎𝑑

The torque–speed characteristic of the selected HP 6V gearmotor is approximated as linear between stall torque and no-load speed. The motor datasheet specifies a no-load speed of 410 rpm and a stall torque of 1.3kg·cm, which corresponds to 0.1274 Nm.

Thus, the torque-speed curve is defined by the two points: (0 rpm, 0.1274 Nm) and (410 rpm, 0 Nm)

The maximum required operating speed of the mixer is 300 rpm. Since torque demand must still be met at this highest intended operating speed, this point is treated as the worst-case operating condition.

𝑛𝑟𝑒𝑞=300 𝑟𝑝𝑚

𝑇𝑟𝑒𝑞=0.01029 𝑁𝑚

This point (300 rpm, 0.02058 Nm) is marked on the diagram as the worst-case operating point. We can see that the worst-case operating point lies below the motor torque-speed curve.